# Maxwell–Jones Vector-Sensor Localization and Synchronization for Distributed Near-Field RIS-Aided OFDM

Qingshan Gong,

Abstract

We estimate a single-antenna user’s 3-D position and common clock offset from its orthogonal frequency-division multiplexing (OFDM) uplink, which reaches one electromagnetic vector-sensor (EVS) array only through distributed near-field reconfigurable intelligent surfaces (RISs). Jones-domain path coefficients and cascade-to-panel association are unknown. A per-panel space–frequency/Maxwell–Jones factorization shows that receiver-component selection rescales local information but can eliminate panel separability. Calibration fixes a Maxwell receive subspace independent of position and clock. In the reference scene, projection before acquisition reduces 96 receive dimensions to 6. Under the matched white-Gaussian model, the profiled maximizer and free-Jones equivalent Fisher information matrix are unchanged. The profiled Gram matrix is clock-invariant. At fixed position, the conditional clock profile is a trigonometric polynomial with a deterministic optimality-gap certificate; the outer search is 3-D. At a signal-to-noise ratio of −10 dB in the matched center-frequency reference scene, the estimator has a 0.32-mm median position error and position and clock catastrophic rates of 0.42% and 0.31%, 6.0- and 6.7-fold lower than the closest in-model control.

Gong: Maxwell–Jones Vector-Sensor Localization and Synchronization

Clock synchronization, Cramér–Rao bound, EVS, near-field localization, OFDM, RIS, variable projection.

# Introduction

the near field of an electrically large reconfigurable intelligent surface (RIS) panel, the phase across the aperture depends on the direction and propagation range of the user equipment (UE) relative to the panel . A surveyed panel can use this variation to constrain the UE position in three dimensions . With the direct link blocked, the base station (BS) observes cascades through distributed panels . Each cascade combines geometric delay with the same UE clock offset; the passive panels supply no independent timing reference.

Distributed-RIS localization combines geometric constraints from multiple panel paths , and several formulations jointly estimate an unknown clock offset . Near-field estimators retain spherical-wave phase across electrically large apertures . Multilinear methods factor the training tensor . With all cascades observed at one array, the factors returned by an algebraic front end still need to be assigned to surveyed panels before the geometric fit. Sparse and variational methods estimate parameters on a discretized manifold ; polar or beam-split codebooks search direction and range . Resolving finer aperture-scale detail with a fixed grid increases the dictionary or search density.

Dual-polarized RISs use polarization for beamforming gain , whereas an electromagnetic vector sensor (EVS) measures co-located electric and magnetic fields to estimate the incident polarization together with its direction . RIS-aided EVS radar extends this processing to joint departure-angle, arrival-angle and polarization estimation . Our previous EVS estimator for orthogonal frequency-division multiplexing (OFDM)  treats a direct multipath channel. The distributed cascades add panel association and a common clock to the geometric fit.

First, distributed-RIS methods typically resolve cascade-to-panel association before or during the joint fit . All cascades reach the same EVS array within one observation, and an algebraic front end returns them unlabeled . Panels with nearly coincident delays then present similar space–frequency evidence, making separability depend on the retained Maxwell structure of the receiver. Second, full-EVS training produces six field components over MA sensors, N subcarriers and T configurations; a general linear reduction need not preserve the delay subspace, profiled likelihood or localization information. Third, differencing cascaded OFDM delays removes the common clock but leaves only K−1 differences for three coordinates and is underdetermined at K=3. Retaining the absolute delays gives a nonconvex 4-D position–clock problem. Variable projection  eliminates the linear nuisances but leaves position and clock coupled. The semidefinite method of  uses a timing model with a linear lifting, which the cascaded OFDM likelihood does not admit.

The sensing block of the kth RIS panel factorizes as

$$\mathbf{A}_k(\boldsymbol{\chi})=
    \underbrace{\mathbf{c}_k(\boldsymbol{\chi})}_{\text{space--frequency geometry}}
    \otimes
    \underbrace{\boldsymbol{\Theta}_k^{({\rm mode})}}_{\text{calibrated Maxwell--Jones}},
    \label{eq:premise}$$

where χ=pT,ΔtT collects the unknown position and clock, ck depends on both, and $\boldsymbol{\Theta}_k^{({\rm mode})}$—two columns for the polarization state, one row per retained field component—is fixed by the calibrated RIS-to-BS geometry and the selected receiver mode.

C1. Receiver-mode information and panel separability. Component selection rescales a panel’s free-Jones equivalent Fisher information matrix (EFIM), with Jones coefficients treated as unknown nuisances, by the retained polarization energy without changing its eigenvectors. Panel separability instead depends on a residual in the retained Maxwell receive subspace; this residual can vanish with no more than two field components even when the local EFIM remains finite (Theorem 1; Figs. 1 and [fig:c1_separability]).

C2. Calibrated sufficient compression. Calibration fixes the Maxwell receive subspace independently of χ. In the reference scene, projecting onto it reduces the receive dimension from 96 to 6. For the matched white-Gaussian model, the projected data preserve the profiled maximizer and free-Jones EFIM exactly. They are sufficient when the variance is known; with unknown variance, sufficiency also requires the discarded energy. In the finite-sample test, removing the other 90 directions improves delay-subspace acquisition (Theorem 2; Table [tab:c2_ladder]).

C3. Certified common-clock profiling. Over equispaced tones, the clock acts through a finite diagonal phase orbit and leaves the profiled Gram matrix invariant. After eliminating the linear coefficients, its conditional profile is a trigonometric polynomial with a deterministic optimality-gap certificate. Eliminating the clock leaves a unit-invariant 3-D outer search (Theorem 3; Table 2).

Gaussian sufficiency for a known signal subspace and elimination of linear parameters by variable projection are classical. Calibration fixes the required subspace without knowing χ. The delay front end reuses the asymmetric tensor folding of ; this paper adds the preceding sufficient compression and the following common-geometry fusion.

The C1 split applies to any array whose response factorizes into a calibrated polarimetric block and a parameter-dependent geometric one. C3 extends to separable least-squares problems in which the nuisance follows an equispaced finite Fourier phase orbit and the profiled Gram matrix is nuisance invariant; under those conditions, the conditional profile remains a trigonometric polynomial that can be maximized globally with a certificate.

Notation: bold symbols denote vectors and matrices; ⋅T, ⋅H, ⋅* and ⋅† are transpose, Hermitian transpose, entrywise complex conjugation and pseudoinverse; ⊗ and ⊙ are Kronecker and Hadamard products; for a generic matrix X, PX=XX† and ΠX⊥=I−PX are the orthogonal projectors onto colX and colX⊥; ∥⋅∥F is the Frobenius norm; and unqualified vector and matrix norms are Euclidean and spectral, respectively.

# Maxwell–Jones Cascaded Signal Model

The signal from a single-antenna UE reaches the BS only through K distributed, arbitrarily oriented RIS panels; the line-of-sight (LoS) path is blocked . Each panel is an MRx×MRy planar array of MR=MRxMRy elements, and the BS is an MA-element EVS uniform linear array (ULA); the UE transmits pilots on N subcarriers spaced by Δf (occupied bandwidth $B_{\rm occ}=(N-1)\Delta f$) over T known RIS phase configurations. The nonlinear parameters of interest are the UE position p and the clock offset Δt. Let fc denote the carrier frequency and c0 the speed of light; then λc=c0/fc is the carrier wavelength.

(A1) The LoS path is blocked and each of the K calibrated panels contributes one dominant cascade, so the model order is known; Section 5.6 quantifies the effect of an incorrect K. (A2) The BS and panel geometries are surveyed, and the EVS Maxwell response and its per-component gains are calibrated ; Fig. [fig:boundaries](a)–(b) sweeps residual calibration error, while receiver-side calibration error is audited in the supplementary material. (A3) Following the one-sided near-field convention of , the UE–RIS hop is modeled with an exact spherical-wave phase over a uniform-amplitude aperture and the RIS–BS hop with a center-frequency plane wave; Section 5.1 verifies $\max_k d_{{\rm UR},k}<R_{\rm FF}<\min_k d_{{\rm RB},k}$, where $R_{\rm FF}=2D_{\rm ap}^2/\lambda_c$ is a panel’s Fraunhofer distance and $D_{\rm ap}$ its aperture diameter, the corner-to-corner span of the element grid. (A4) Spatial responses are evaluated at fc and the wideband dependence is confined to the delay phase ; the neglected spatial-wideband phase reaches $\epsilon_{\rm sq}=0.102$ rad in the reference scene. Section 5.1 diagnoses the resulting bias. (A5) The cyclic prefix covers the cascaded delay spread and one common clock offset enters every delay; residual carrier-frequency offset, phase noise and panel-dependent timing offsets  lie outside the model. (A6) UE position, clock, gains and Jones states are block-static over the T configurations and the noise is white with unknown variance; Section 5.6 reports the coherence budget; a colored-noise sweep is supplementary.

The received signal is

y=k=1Khkp⏟near field⊗dkp,Δt⏟delay⊗Bk⏟calibratedxk+n,

with y,n∈CM, M=I N T for the receive dimension I of Section 2.1, and xk∈C2 a free Jones coefficient; only hk and dk depend on the unknown p,Δt.

image

## Calibrated RIS–BS Maxwell Factor

Calibration provides pB, the BS phase center; rk and $\mathbf{R}_k\in{\rm SO}(3)$, the center and the global-to-local rotation of panel k; ρk,m∈R3, its element coordinates in that frame; and the BS array geometry and EVS Maxwell response. The BS ULA lies along the global x axis of the orthonormal frame {ux,uy,uz} and κ=2π/λc is the center-frequency wavenumber.

The calibrated locations give the RIS–BS distance and propagation direction of panel k,

$$d_{{\rm RB},k}=\|\mathbf{p}_B-\mathbf{r}_k\|_2,
    \qquad
    \mathbf{s}_{{\rm RB},k}=\frac{\mathbf{p}_B-\mathbf{r}_k}{d_{{\rm RB},k}},
    \label{eq:rbgeom}$$

and, under the far-field RIS–BS model established above, a plane-wave panel response and a BS steering vector,

$$\begin{aligned}
    \big[\mathbf{a}_{{\rm RB},k}\big]_m
    &=e^{j\kappa\,\boldsymbol{\rho}_{k,m}^{\mathsf T}\mathbf{R}_k\mathbf{s}_{{\rm RB},k}},\nonumber\\
    \big[\mathbf{v}_{{\rm B},k}\big]_i
    &=e^{-j\kappa x_i\,\mathbf{u}_x^{\mathsf T}\mathbf{s}_{{\rm RB},k}},
    \label{eq:arb_vb}
\end{aligned}$$

with xi the x coordinate of the ith BS element, $\mathbf{a}_{{\rm RB},k}\in\mathbb{C}^{M_R}$ and $\mathbf{v}_{{\rm B},k}\in\mathbb{C}^{M_A}$.

The same direction fixes the polarization manifold: a transverse plane wave along $\mathbf{s}_{{\rm RB},k}$ has Maxwell pair $(\mathbf{E},\mathbf{H})=(\mathbf{q},\mathbf{s}_{{\rm RB},k}\times\mathbf{q})$ in impedance-normalized units, so with any transverse basis

$$\mathbf{q}_{k,1}\in\mathbf{s}_{{\rm RB},k}^{\perp},
    \quad \|\mathbf{q}_{k,1}\|_2=1,
    \qquad
    \mathbf{q}_{k,2}=\mathbf{s}_{{\rm RB},k}\times\mathbf{q}_{k,1},
    \label{eq:qbasis}$$

the six-component vector-sensor manifold  specializes to the calibrated Maxwell–Jones block

Θk6=qk,1qk,2qk,2−qk,1∈C6×2,

whose columns satisfy Θk6HΘk6=2I2; a rotation $\mathbf{U}\in{\rm SO}(2)$ of the transverse basis sends Θk6↦Θk6U and is absorbed by xk↦UTxk. A fixed selection matrix $\mathbf{S}_{\rm mode}\in\mathbb{R}^{d_{\rm evs}\times6}$,

S1=10, S2=I20, S6=I6,

with $d_{\rm evs}\in\{1,2,6\}$ for the scalar, dual-polarized and full-EVS receiver modes of Fig. [fig:geometry](c), gives $\boldsymbol{\Theta}_k^{({\rm mode})}\triangleq\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_k^{(6)}
\in\mathbb{C}^{d_{\rm evs}\times2}$ and the receive dimension $I=d_{\rm evs}M_A$.

The calibrated receive block $\mathbf{B}_k\triangleq\mathbf{v}_{{\rm B},k}\otimes\boldsymbol{\Theta}_k^{({\rm mode})}\in\mathbb{C}^{I\times2}$ is known before UE estimation and satisfies ∂Bk/∂χ=0; only the Jones-domain coefficients xk∈C2 of [eq:roadmap] remain unknown, each being the effective polarization state launched toward the BS after reflection rather than the UE antenna state.

## UE–RIS Near-Field and Common-Clock Factor

The unknown nonlinear parameter collects the UE position and common clock offset, χ=pT,ΔtT=x,y,z,ΔtT∈R4, with p∈P a known closed service box and Δt∈T, where T is a known closed interval with T<1/Δf, the unambiguous delay period. Every cascaded delay is the geometric propagation time plus the same offset . In the kth panel frame (Fig. [fig:geometry](b)), ξkp=Rkp−rk and

$$d_{{\rm UR},k}(\mathbf{p})=\|\boldsymbol{\xi}_k(\mathbf{p})\|_2=\|\mathbf{p}-\mathbf{r}_k\|_2,
    \quad
    \mathbf{s}_{{\rm UR},k}(\mathbf{p})=\frac{\boldsymbol{\xi}_k(\mathbf{p})}{d_{{\rm UR},k}(\mathbf{p})},
    \label{eq:urgeom}$$

since Rk is a rotation. Element m of panel k applies the scalar Jones response ejωk,t,mI2 at configuration t, so no polarization mixing occurs; with the prescribed constant-modulus training Ωkt,m=ejωk,t,m/MR, Ωk∈CT×MR , the near-field dependence is captured by the RIS response

$$\begin{aligned}
    \mathbf{g}_k(\mathbf{p}) &= \mathbf{a}_{{\rm RB},k}\odot\mathbf{a}^{\rm NF}_{{\rm UR},k}(\mathbf{p})
    \in\mathbb{C}^{M_R},\nonumber\\
    \mathbf{h}_k(\mathbf{p}) &= \boldsymbol{\Omega}_k\,\mathbf{g}_k(\mathbf{p})\in\mathbb{C}^{T},
    \label{eq:hk}
\end{aligned}$$

where

$$\begin{aligned}
    \big[\mathbf{a}^{\rm NF}_{{\rm UR},k}(\mathbf{p})\big]_m
    &= e^{-j\kappa\delta_{k,m}(\mathbf{p})},
    \quad \mathbf{a}^{\rm NF}_{{\rm UR},k}\in\mathbb{C}^{M_R},\nonumber\\
    \delta_{k,m}(\mathbf{p})
    &= \big\|\boldsymbol{\xi}_k(\mathbf{p})-\boldsymbol{\rho}_{k,m}\big\|_2
    - d_{{\rm UR},k}(\mathbf{p}).
    \label{eq:delta}
\end{aligned}$$

Equation [eq:delta] retains the exact spherical-wave phase across the panel aperture (Fig. [fig:geometry](b)); the Fresnel approximation enters only in Phase-I proposal generation (Section 3.2).

Combining [eq:rbgeom] and [eq:urgeom], the cascaded delay and its OFDM phase ramp on the tone grid fn=fc+n−N−12Δf, n=0,…,N−1, are

$$\begin{aligned}
    \tau_k(\mathbf{p},\Delta t)
    &= \underbrace{\frac{d_{{\rm UR},k}(\mathbf{p}) + d_{{\rm RB},k}}{c_0}}_{
    \textstyle \tau_{g,k}(\mathbf{p})} + \Delta t,\nonumber\\
    \mathbf{d}_k(\mathbf{p},\Delta t)
    &= \Big[\,e^{-j2\pi\left(n-\frac{N-1}{2}\right)\Delta f\,
    \tau_k(\mathbf{p},\Delta t)}\,\Big]_{n=0}^{N-1}\in\mathbb{C}^N,
    \label{eq:dk}
\end{aligned}$$

with the tone-independent phase e−j2πfcτk absorbed into the path gain. In contrast to the frequency-dependent spatial manifold of , the present model carries the wideband dependence entirely through [eq:dk]. For an inter-element path difference ΔL in a spatial response, the phase neglected in (A4) is $\epsilon_{\rm sq}=\pi B_{\rm occ}\max|\Delta L|/c_0$.

The bracketed factor of [eq:roadmap] is the per-panel sensing block, which by associativity of the Kronecker product also takes the form [eq:premise],

$$\begin{aligned}
    \mathbf{A}_k(\boldsymbol{\chi})
    &= \mathbf{h}_k(\mathbf{p})\otimes\mathbf{d}_k(\mathbf{p},\Delta t)\otimes\mathbf{B}_k\nonumber\\
    &= \mathbf{c}_k(\boldsymbol{\chi})\otimes\boldsymbol{\Theta}_k^{({\rm mode})}\in\mathbb{C}^{M\times2},
    \label{eq:Ak}
\end{aligned}$$

where $\mathbf{c}_k(\boldsymbol{\chi})=\mathbf{h}_k(\mathbf{p})\otimes\mathbf{d}_k(\mathbf{p},\Delta t)\otimes
\mathbf{v}_{{\rm B},k}\in\mathbb{C}^{TNM_A}$ is the space–frequency factor.

Arranging the I receive channels against the N subcarriers at configuration t gives

Yt=k=1Khkpt Bkxk dkp,ΔtT+Nt,

where t=1,…,T, Yt∈CI×N and Nt is the noise block. The grouping hk⊗dk⊗Bk exposes the receive subspace used in Section 3; $\mathbf{c}_k\otimes\boldsymbol{\Theta}_k^{({\rm mode})}$ isolates receiver-mode selection for Section 4.

## Observation and Estimation Problem

Stacking the panels as Aχ=A1χ,…,AKχ∈CM×2K and vectorizing the Yt in the same order writes [eq:roadmap] as

y=μχ,x+n=Aχ x+n,  n∼CN0,σ2IM,

where the noise is white across receive channels, subcarriers and RIS configurations, and its power σ2 is unknown to the estimator. The unknown Jones-domain coefficient of path k is xk=βkek∈C2, stacked as x=x1T,…,xKTT∈C2K, where the complex gain βk absorbs path loss, RIS efficiency and the center-frequency phase of the cascade and ek is a unit Jones state (angular parameterization in Table 1). Both factors are flat over the occupied band and only their product is identifiable, so the estimator treats xk as a free C2 nuisance.

Profiling σ2 out of the white-Gaussian log-likelihood gives σ2=∥y−Aχx∥22/M, and the concentrated log-likelihood is strictly decreasing in the residual energy, so maximum-likelihood (ML) estimation of χ,x is the least-squares problem

minp∈P, Δt∈T, x∈C2K∥y−Ap,Δt x∥22.

The 2K Jones coefficients enter linearly and admit closed-form elimination; position and clock enter nonlinearly through [eq:delta] and [eq:dk], leaving [eq:ml_problem] nonconvex in four real coordinates.

# Proposed Estimator: Sufficient Acquisition and Certified Clock Profiling

Direct optimization of [eq:ml_problem] must address panel association, near-field basin acquisition, the 2K linear Jones coefficients and the common clock simultaneously. We separate them with a two-phase estimator. Phase I, Maxwell–Kronecker sufficient compression with common-geometry initialization (MKSC-GI), projects onto the calibrated Maxwell receive subspace, recovers and associates the cascaded factors, and fuses the panel-local estimates into one common position initializer. Phase II, common-clock orbit-profiled Jones variable projection (CCOP-JVP), eliminates the Jones coefficients analytically and refines position and clock by certified profiling along the exact clock orbit. Algorithm [alg:mksc_ccop] collects both phases, with each step citing its corresponding derivation. In the algorithm, ESPRIT denotes estimation of signal parameters via rotational invariance techniques; DFT and FFT denote the discrete and fast Fourier transforms, respectively.

{Yt}t=1T, calibrated infrastructure, K, P, T Phase I: MKSC-GI Compress receive index: $\mathbf{Y}_{{\rm d},t}=\mathbf{Q}_S^{\mathsf H}\mathbf{Y}_t$, I​→​r≤2K [eq:calibsub] Delay poles {zk} by ESPRIT on the tone axis [eq:shift] Coupled factors $\hat\mathbf{F}$ by one loaded weighted least-squares (WLS) solve [eq:coupledls_short] Rank-one split $\to(\tilde\mathbf{a}_j,\tilde\mathbf{t}_j,\varrho_j)$ [eq:split_rank1] Dechirped 2-D DFT proposals for all panel–column pairs [eq:fresnel], [eq:Jris_short] Two-tier permutation search with exact shortlist rescoring [eq:assoc_perm] Fuse by $J_{\rm geom}$; select by clock spread [eq:Jgeom_short] Jones-state anchors $\{\hat\mathbf{e}_k\}$ refreshed at $\hat\mathbf{p}_{\rm GI}$ [eq:refresh_short] Select Phase-II position initializer $\hat\mathbf{p}_{\rm I}$ by clock–geometry consistency Phase II: CCOP-JVP Clock-profile coefficients {γℓp}; FFT-seeded candidates [eq:clock_coeff] Branch and bound over Δt until $g_{\rm cert}\!\le\!\epsilon_{\rm clk}$ or cap [eq:bound_short] Update p by envelope gradient (safeguard if ambiguous) [eq:envelope] Profiled Jones coefficients $\hat\mathbf{x}=\mathbf{G}_\lambda^{\dagger}\mathbf{b}$ [eq:vp_short] $(\hat\mathbf{p},\widehat{\Delta t},\hat\mathbf{x})$ and $g_{\rm cert}$

## Maxwell–Kronecker Factor Recovery

1) Sufficient compression and delay-pole recovery: The calibrated receive blocks in [eq:Ak] span

SR=k=1KcolBk=colΨ, Ψ=B1,…,BK,

with r=rankΨ≤2K. An orthonormal basis QS∈CI×r depends only on calibration and satisfies QSQSHBk=Bk for every panel. Projection onto this basis gives

$$\mathbf{Y}_{{\rm d},t}=\mathbf{Q}_S^{\mathsf H}\mathbf{Y}_t
 =\sum_{k=1}^{K}[\mathbf{h}_k(\mathbf{p})]_t\,\mathbf{a}_k\,
 \mathbf{d}_k(\mathbf{p},\Delta t)^{\mathsf T}+\mathbf{Q}_S^{\mathsf H}\mathbf{N}_t,
 \label{eq:obsmat_c}$$

where ak=QSHBkxk. Theorem 2 establishes sufficiency. In the reference scene, the projection reduces the receive dimension from I=96 to r=6, discarding noise-only directions while retaining white noise in the compressed coordinates.

Each delay contributes the pole zk=e−j2πΔfτk. The centered tone grid adds only a nonzero column scalar: dkn=ejπN−1Δfτkzkn. Rearranging the compressed channel–configuration pairs as snapshots gives

Yf=k=1Kdk wkT+Nf∈CN×rT,  wk=ak⊗hkp.

For D=d1,…,dK, removing its last or first row gives D or D, respectively, with

D=D Φ,  Φ=diagz1,…,zK.

Distinct delays modulo 1/Δf, N≥K+1 and the snapshot-rank condition of Theorem 2(iv) permit recovery by forward–backward total-least-squares ESPRIT , with estimated poles projected onto the unit circle.

2) Coupled factor recovery: With the estimated delays fixed, we recover F=w1,…,wK∈CrT×K from the rearranged data YrT,N by

$$\hat\mathbf{F}=\arg\min_{\mathbf{F}}
 \big\|(\mathbf{Y}_{rT,N}-\mathbf{F}\mathbf{D}^{\mathsf T})\mathbf{W}^{1/2}\big\|_F^2
 +\lambda_\tau\|\mathbf{F}\|_F^2,
 \label{eq:coupledls_obj}$$

or, equivalently, one K×K solve:

$$\hat\mathbf{F}=\mathbf{Y}_{rT,N}\mathbf{W}\mathbf{D}^{\ast}
 (\mathbf{D}^{\mathsf T}\mathbf{W}\mathbf{D}^{\ast}+\lambda_\tau\mathbf{I})^{-1}.
 \label{eq:coupledls_short}$$

The diagonal entries of W=diagwn are the anti-diagonal multiplicities wn=#{p,ℓ:p+ℓ=n} of the P×L Hankel lift of the tone axis (P+L−1=N); solving in the lifted domain gives the same linear system. The window weights this factor recovery only: the poles of [eq:shift] are estimated from the full N-tone aperture, unsmoothed. After column j is reshaped as an r×T matrix with the receive index running slowest, its leading singular triplet gives

$$\tilde\mathbf{a}_j=\sqrt{\sigma_{1,j}}\,\mathbf{Q}_S\mathbf{u}_{1,j},\quad
 \tilde\mathbf{t}_j=\sqrt{\sigma_{1,j}}\,\mathbf{v}_{1,j}^{\ast},\quad
 \varrho_j=\frac{\sigma_{2,j}}{\sigma_{1,j}}.
 \label{eq:split_rank1}$$

For exact poles and zero loading, the noiseless factors of the generating panel k satisfy

$$\tilde\mathbf{a}_j=c_j\,\mathbf{B}_k\mathbf{x}_k,\qquad
 \tilde\mathbf{t}_j=c_j'\,\mathbf{h}_k(\mathbf{p}),
 \label{eq:factor_meaning}$$

where cj,cj′≠0 encode a scale and phase ambiguity, so subsequent criteria must be scale invariant; 𝜚j measures departure from rank one and weights the common-geometry fit.

3) Physical panel association: For column j and candidate panel k, the score combines calibrated receive-subspace and spherical-wave consistency:

$$s_{j,k}=
 \frac{\|\boldsymbol{\Pi}^{\perp}_{\mathbf{B}_k}\tilde\mathbf{a}_j\|}{\|\tilde\mathbf{a}_j\|}
 +\min_{\boldsymbol\eta\in\mathcal E_k}
 \frac{\|\boldsymbol{\Pi}^{\perp}_{\mathbf{h}_k(\boldsymbol\eta)}\tilde\mathbf{t}_j\|}
 {\|\tilde\mathbf{t}_j\|}.
 \label{eq:assoc_score}$$

The vector η=d,u,v contains a candidate range and the first two direction cosines in panel k’s local frame; Ek is its search box, and hk is the exact response [eq:hk]. The minimization over Ek is initialized by the beam-space proposals of Section 3.2 and is evaluated for all K2 panel–column pairs before an association is fixed. The minimizing range dj,k implies a clock replica $\widehat{\Delta t}_{j,k}=\hat\tau_j-(\hat d_{j,k}+d_{{\rm RB},k})/c_0$. Each permutation is scored by

$$C(\pi)=\sum_{j=1}^{K}s_{j,\pi(j)}
 +\frac{w_{\rm clk}}{t_0}
 [\,\mathrm{sd}(\pi)+e_{\mathcal T}(\pi)\,],
 \label{eq:assoc_perm}$$

where eT is the mean excursion outside T, $w_{\rm clk}=1/2$, t0=1 ns, and

sdπ=[1Kj=1KΔtj,πj−Δtπ2]1/2,Δtπ=1Kj=1KΔtj,πj.

The common clock mean Δtπ couples the assignments, so this is not a linear assignment problem. A deterministic two-tier search ranks all K! permutations using a cached coarse codebook, then re-scores only the best two, π1,π2, using exact spherical projection:

π=argminπ∈{π1,π2}Cπ.

It returns the delay–factor triples $\{\hat\tau_j,\tilde\mathbf{a}_j,\tilde\mathbf{t}_j;\hat\pi(j)\}$ indexed by their associated panels for geometric acquisition.

## Beam-Space Acquisition and Common-Geometry Initialization

1) Panel-local beam-space acquisition: By [eq:factor_meaning] the factor $\tilde\mathbf{t}_k$ that π assigns to panel k is hk up to an unknown scale. Its geometry is recovered separately for each panel before the K estimates are reconciled into a common position by the scale-invariant fit

$$J_{\rm RIS}(\boldsymbol\eta)
 =\|\boldsymbol{\Pi}_{\mathbf{h}_k(\boldsymbol\eta)}^\perp\tilde\mathbf{t}_k\|^2
 =\|\tilde\mathbf{t}_k\|^2-
 \frac{|\mathbf{h}_k^{\mathsf H}(\boldsymbol\eta)\tilde\mathbf{t}_k|^2}
 {\|\mathbf{h}_k(\boldsymbol\eta)\|^2},
 \label{eq:Jris_short}$$

with $\mathbf{h}_k(\boldsymbol\eta)=\boldsymbol{\Omega}_k[\mathbf{a}_{{\rm RB},k}\odot
\mathbf{a}^{\rm NF}(\boldsymbol\eta)]$ over the panel-local coordinates η defined above. A fixed-count angular dictionary of the polar or beam-training kind  has a step size set by the search box, not by the panel resolution $\lambda_c/D_{\rm ap}$, so the dominant angular search is moved to the element domain. The training code enters hk only on the left, so moving ΩkH across and splitting the Hadamard product gives

$$\begin{aligned}
 \mathbf{h}_k^{\mathsf H}(\boldsymbol\eta)\tilde\mathbf{t}_k
 &=\big[\mathbf{a}_{{\rm RB},k}\odot\mathbf{a}^{\rm NF}(\boldsymbol\eta)\big]^{\mathsf H}
 \big(\boldsymbol{\Omega}_k^{\mathsf H}\tilde\mathbf{t}_k\big)
 =\big[\mathbf{a}^{\rm NF}(\boldsymbol\eta)\big]^{\mathsf H}\tilde\mathbf{g}_k,\nonumber\\
 \tilde\mathbf{g}_k&=\mathbf{a}_{{\rm RB},k}^{\ast}\odot
 (\boldsymbol{\Omega}_k^{\mathsf H}\tilde\mathbf{t}_k).
 \label{eq:adjoint_short}
\end{aligned}$$

One adjoint pass fixes $\tilde\mathbf{g}_k$ per panel and leaves $\mathbf{a}^{\rm NF}(\boldsymbol\eta)$ as the only η-dependent factor in the numerator of [eq:Jris_short]; the denominator admits no such reduction and is evaluated exactly. Expanding [eq:delta] to second order about the panel center, with u=u,v,1−u2−v2T the unit direction from that center to the UE in the panel frame,

δk,m≃−ρk,mTu+∥ρk,m∥2−ρk,mTu22d,

so the paraxial dechirp $\tilde\mathbf{g}_{k,\tilde d}[m]=\tilde\mathbf{g}_k[m]
e^{j\kappa\|\boldsymbol{\rho}_{k,m}\|^2/(2\tilde d)}$ removes the isotropic quadratic term at trial range d and leaves the plane-wave phase ejκρk,mTu, up to the direction-dependent curvature ρk,mTu2/2d. One zero-padded 2-D DFT then scores all direction-cosine pairs at once on a grid tracking the Rayleigh limit, requiring one transform per trial range rather than an atom-by-atom dictionary scan. Because of the residual curvature, [eq:fresnel] is used only to generate proposals: every retained angular peak receives an exact spherical range scan and is re-scored by the full [eq:Jris_short], so the candidate set changes acquisition coverage and cost but not the forward model. The candidate set for each experiment is stated in Section 5.

2) Common-geometry fusion: Each panel-local estimate ηk maps through the calibrated panel pose back to a global position $\hat\mathbf{p}_k^{\rm loc}=\mathbf{r}_k+\mathbf{R}_k^{\mathsf T}\boldsymbol{\xi}_k(\hat{\boldsymbol\eta}_k)$, inverting ξkp=Rkp−rk. These K positions generally disagree; the common-geometry stage replaces them by the single position that best explains all K training factors at once, each up to its own unknown complex scale,

$$\begin{aligned}
 J_{\rm geom}(\mathbf{p})
 &=\min_{\boldsymbol{\alpha}\in\mathbb{C}^K}\sum_{k=1}^K \varpi_k
 \|\tilde\mathbf{t}_k-\alpha_k\mathbf{h}_k(\mathbf{p})\|^2
 \label{eq:Jgeom_short}\\
 &=\sum_{k=1}^K \varpi_k\left[\|\tilde\mathbf{t}_k\|^2-
 \frac{|\mathbf{h}_k^{\mathsf H}(\mathbf{p})\tilde\mathbf{t}_k|^2}{\|\mathbf{h}_k(\mathbf{p})\|^2}\right],
 \nonumber
\end{aligned}$$

the second line following from the profiled scale $\hat\alpha_k(\mathbf{p})=\mathbf{h}_k^{\mathsf H}(\mathbf{p})\tilde\mathbf{t}_k/\|\mathbf{h}_k(\mathbf{p})\|^2$, so only the component of $\tilde\mathbf{t}_k$ orthogonal to colhk remains and the gains need not be known. The weights ϖk=max{e−𝜚k/𝜚0,ϖmin} down-weight a column whose rank-one split is poor while keeping every panel in the fit. We minimize [eq:Jgeom_short] with bounded L-BFGS-B  from deterministic starts around $\hat\mathbf{p}_0=\operatorname{clip}_{\mathcal P}\{\operatorname{median}_k\hat\mathbf{p}_k^{\rm loc}\}$. Among the near-optimal minima, we select $\hat\mathbf{p}_{\rm GI}$ as the one whose clock replicas $\hat\tau_k-(\|\mathbf{p}-\mathbf{r}_k\|+d_{{\rm RB},k})/c_0$ vary least.

3) Jones-state anchor refresh: At the selected $\hat\mathbf{p}_{\rm GI}$, the λk=0 instance of the certified clock profiler of Section 3.3 refreshes the Jones-state anchors at a fixed position,

$$(\widehat{\Delta t}_{\rm ref},\hat\mathbf{x}_{\rm ref})=
 \arg\min_{\Delta t\in\mathcal T,\mathbf{x}}
 \|\mathbf{y}-\mathbf{A}(\hat\mathbf{p}_{\rm GI},\Delta t)\mathbf{x}\|^2.
 \label{eq:refresh_short}$$

With $\hat\mathbf{x}_{{\rm ref},k}$ the kth two-entry block of $\hat\mathbf{x}_{\rm ref}$, the anchor is $\hat\mathbf{e}_k=\hat\mathbf{x}_{{\rm ref},k}/\|\hat\mathbf{x}_{{\rm ref},k}\|_2$, obtained at the common position from a conditional clock solve with an optimality-gap certificate of the same form. The implementation chooses the Phase-II position initializer $\hat\mathbf{p}_{\rm I}$ from $\hat\mathbf{p}_{\rm GI}$, the mean and coordinatewise median of the panel-local positions, and their K leave-one-out means by minimizing the median absolute deviation of the implied panel clocks plus the median position discrepancy divided by c0. The refreshed anchors initialize the Phase-II directional regularizer.

## Common-Clock Orbit-Profiled Jones Variable Projection

1) Unitary clock orbit and directional Jones regularization: Phase II refines the Phase-I initializer using the exact forward model underlying [eq:ml_problem] and a directional Jones penalty. Its 2K Jones coefficients enter linearly and are eliminated in closed form by variable projection, leaving four nonlinear coordinates. By Theorem 2, the implementation evaluates the criterion on the compressed observation of Phase I without approximation. The common clock is the only nonlinear coordinate along which the likelihood admits an exact unitary symmetry. Let A0p≜Ap,0 and $\mathbf{D}_{\rm clk}(\Delta t)=\operatorname{diag}([e^{-j2\pi(n-\frac{N-1}{2})\Delta f\Delta t}]_{n=0}^{N-1})$. Then

$$\begin{aligned}
 \mathbf{U}_{\rm clk}(\Delta t)&=\mathbf{I}_T\otimes\mathbf{D}_{\rm clk}(\Delta t)\otimes\mathbf{I}_I,
 \nonumber\\
 \mathbf{A}(\mathbf{p},\Delta t)&=\mathbf{U}_{\rm clk}(\Delta t)\mathbf{A}_0(\mathbf{p}),
 \label{eq:orbit_short}\\
 \mathbf{A}^{\mathsf H}\mathbf{A}
 &=\mathbf{A}_0^{\mathsf H}\mathbf{U}_{\rm clk}^{\mathsf H}\mathbf{U}_{\rm clk}\mathbf{A}_0\nonumber\\
 &=\mathbf{A}_0^{\mathsf H}\mathbf{A}_0\triangleq\mathbf{G}_0(\mathbf{p}).
 \label{eq:clock_gram}
\end{aligned}$$

Phase II uses the clock-independent, Gram-scaled penalty

$$\mathbf{R}_{\rm dir}(\mathbf{p})=\operatorname{blkdiag}_k\left[
 \lambda_k\tfrac12\operatorname{tr}\mathbf{G}_{0,kk}(\mathbf{p})\,\boldsymbol{\Pi}_{\hat\mathbf{e}_k}^{\perp}\right],
 \label{eq:Rdir_short}$$

with λk=1 throughout (Remark 2) and $\mathbf{R}_{{\rm dir},kk}\hat\mathbf{e}_k=\mathbf0$, leaving the anchor direction and complex gain free. The diagonal data blocks satisfy

$$\mathbf{G}_{0,kk}=N M_A\|\mathbf{h}_k(\mathbf{p})\|^2\,
    [\boldsymbol{\Theta}_k^{({\rm mode})}]^{\mathsf H}\boldsymbol{\Theta}_k^{({\rm mode})}.
    \label{eq:g0block}$$

The receiver mode determines how many Jones coordinates are observable,

$$\operatorname{rank}\mathbf{G}_{0,kk}=\operatorname{rank}\boldsymbol{\Theta}_k^{({\rm mode})}
    =\min\{d_{\rm evs},2\},
    \label{eq:g0rank}$$

for generic panel orientations. The scalar receiver mode leaves one Jones coordinate per panel unobserved. For the full normal matrix $\mathbf{G}_\lambda=\mathbf{G}_0+\mathbf{R}_{\rm dir}$, positive definiteness is characterized by

$$\mathbf{G}_\lambda\succ\mathbf0
    \iff
 \ker(\mathbf{G}_0)\cap\ker(\mathbf{R}_{\rm dir})=\{\mathbf0\}.
    \label{eq:glambda_pd}$$

2) Jones variable projection and the trigonometric clock profile: Phase II accordingly minimizes the penalized exact-model criterion

$$J(\mathbf{p},\Delta t,\mathbf{x})=\|\mathbf{y}-\mathbf{A}(\mathbf{p},\Delta t)\mathbf{x}\|_2^2
 +\mathbf{x}^{\mathsf H}\mathbf{R}_{\rm dir}(\mathbf{p})\mathbf{x}.
 \label{eq:penalized_short}$$

Theorem 3 establishes exact nesting of this criterion, which reduces to [eq:ml_problem] at λk=0. Substituting [eq:orbit_short] into [eq:penalized_short] and using $\mathbf{U}_{\rm clk}^{\mathsf H}\mathbf{U}_{\rm clk}=\mathbf{I}$ makes J a quadratic in x whose Hessian is independent of the clock,

J=∥y∥22−2Re{xHbp,Δt}+xHGλpx,

with $\mathbf{b}=\mathbf{A}_0^{\mathsf H}\mathbf{U}_{\rm clk}^{\mathsf H}\mathbf{y}$. The Wirtinger stationarity condition ∂J/∂x*=Gλx−b=0 removes all 2K Jones coefficients in one step; for Gλ≻0,

$$\begin{aligned}
 \hat\mathbf{x}&=\mathbf{G}_\lambda^{-1}\mathbf{b},
 \quad Q(\mathbf{p},\Delta t)=\mathbf{b}^{\mathsf H}\mathbf{G}_\lambda^{-1}\mathbf{b},\nonumber\\
 F(\mathbf{p},\Delta t)&\triangleq\min_{\mathbf{x}\in\mathbb{C}^{2K}}J=\|\mathbf{y}\|_2^2-Q.
 \label{eq:vp_short}
\end{aligned}$$

At singular positions, the implementation uses Gλ†b as the minimum-norm Jones solution, without adding a ridge. Since b∈colGλ, this preserves the fixed-position criterion and the polynomial clock profile below. The 3-D outer objective is

Fp=∥y∥2−maxΔt∈TQp,Δt.

We group the sufficient-statistic correlations by tone as bp,Δt=n=0N−1unpej2πn−N−12ΔfΔt. Because Gλ−1 is clock-independent, substitution into Q gives

Qp,Δt=n=0N−1n′=0N−1unHGλ−1un′ ej2πn′−nΔfΔt=ℓ=−N−1N−1γℓpej2πℓΔfΔt, γ−ℓ=γℓ*,γℓp=n=0N−1−ℓunHpGλ−1pun+ℓp,

for ℓ=0,…,N−1. This representation makes Q a real trigonometric polynomial of degree at most N−1. The factorization of Gλ at each position is reused for coefficient assembly and FFT-seeded clock candidates; subsequent interval tests evaluate only the polynomial and its derivatives. At fixed p, write qps=Qp,s for clock offset s. Differentiating twice gives

qp″s≤2πΔf2ℓ=−N−1N−1ℓ2γℓ≜L2.

For an interval centered at sc with half-width rs, Taylor expansion yields

qps≤qpsc+qp′scrs+12L2rs2.

3) Certified clock maximization and outer refinement: Branch-and-bound maintains [eq:bound_short] over T and returns an incumbent with $q_{\mathbf{p}}(\Delta t^\star(\mathbf{p}))-q_{\mathbf{p}}(\widehat{\Delta t}(\mathbf{p}))
\le U-q_{\mathbf{p}}(\widehat{\Delta t}(\mathbf{p}))
\triangleq g_{\rm cert}$, where $U=\max\{U_{\rm act},U_{\rm pru}\}$ is the largest bound over surviving and incumbent-pruned intervals. Subdivision stops once $g_{\rm cert}\le\epsilon_{\rm clk}$ or a cap on subdivisions is reached, with the scale-aware tolerance specified in the supplementary material. At every outer position, the returned clock solution has a deterministic gap certificate and is $\epsilon_{\rm clk}$-optimal whenever termination is by tolerance, as Theorem 3(iii) states. For a unique interior maximizer Δt⋆p of Qp,⋅, the exact outer profile satisfies

∇pFp=−∇pQp,Δt|Δt=Δt⋆p.

The outer position search starts from $\hat\mathbf{p}_{\rm I}$ and approximates the envelope gradient by evaluating the right-hand side of [eq:envelope] at the computed Δtp. A derivative-free safeguard is used when the clock solve does not meet its tolerance or the maximizing branch is ambiguous. Phase II outputs $(\hat\mathbf{p},\widehat{\Delta t},\hat\mathbf{x})$ together with a gap certificate for each conditional clock solve evaluated.

# Structural Analysis and Guarantees

We first state the reference free-Jones EFIM and its local rank condition, then establish C1–C3 under their stated conditions.

## Reference Information and Identifiability

Let Dχ=∂Ax/∂χii=14. Eliminating the unknown complex Jones coefficients from the white-Gaussian Fisher information matrix gives the free-Jones equivalent Fisher information matrix (EFIM), the reference information of the unpenalized data model,

$$\mathbf{J}_\chi^{\rm free}=\frac{2}{\sigma^2}
 \operatorname{Re}\{\mathbf{D}_\chi^{\mathsf H}\boldsymbol{\Pi}_{\mathbf{A}}^{\perp}\mathbf{D}_\chi\}.
 \label{eq:efim_short}$$

The projector has a direct interpretation: a geometric perturbation Dχ δχ with δχ∈R4 is indistinguishable from a Jones perturbation exactly when it lies in colA, that is when ΠA⊥Dχ δχ=0. The first-order condition for local position–clock identifiability is obtained by stacking the real and imaginary parts of that map:

$$\widetilde\mathbf{D}_\chi=\begin{bmatrix}
 \operatorname{Re}\{\boldsymbol{\Pi}_{\mathbf{A}}^{\perp}\mathbf{D}_\chi\}\\
 \operatorname{Im}\{\boldsymbol{\Pi}_{\mathbf{A}}^{\perp}\mathbf{D}_\chi\}\end{bmatrix},
 \qquad \operatorname{rank}_{\mathbb{R}}\widetilde\mathbf{D}_\chi=4.
 \label{eq:rankcond}$$

Since $\widetilde\mathbf{D}_\chi^{\mathsf T}\widetilde\mathbf{D}_\chi
=\operatorname{Re}\{\mathbf{D}_\chi^{\mathsf H}\boldsymbol{\Pi}_{\mathbf{A}}^{\perp}\mathbf{D}_\chi\}$, this condition is equivalent to $\mathbf{J}_\chi^{\rm free}\succ\mathbf0$. The EFIM then yields the position error bound (PEB) and clock error bound (CEB), defined as ${\rm PEB}=\sqrt{\operatorname{tr}[(\mathbf{J}_p^{\rm eff})^{-1}]}$ and ${\rm CEB}=\sqrt{[(\mathbf{J}_\chi^{\rm free})^{-1}]_{4,4}}$, with $\mathbf{J}_p^{\rm eff}=\mathbf{J}_{pp}-\mathbf{J}_{pt}J_{tt}^{-1}\mathbf{J}_{tp}$. The blocks Jpp∈R3×3, Jpt=JtpT∈R3×1, and Jtt∈R are the position, cross and clock blocks of $\mathbf{J}_\chi^{\rm free}$. The Jacobian of the cascaded delays in [eq:dk] is

$$\mathbf{D}_\tau(\mathbf{p})
    =\frac{\partial[\tau_1,\ldots,\tau_K]^{\mathsf T}}
    {\partial\boldsymbol{\chi}^{\mathsf T}}
    =\begin{bmatrix}
    c_0^{-1}\mathbf{s}_{{\rm UR},1}^{\mathsf T}\mathbf{R}_1 & 1\\
    \vdots & \vdots\\
    c_0^{-1}\mathbf{s}_{{\rm UR},K}^{\mathsf T}\mathbf{R}_K & 1
    \end{bmatrix}\in\mathbb{R}^{K\times4}.
    \label{eq:delayjac}$$

Row differencing removes the all-ones clock column. At K=3, rankDτ≤3<4, so the cascaded delays cannot identify χ by themselves. Additional position sensitivity comes from the aperture phase of [eq:delta]: with sk,m=ξk−ρk,m/∥ξk−ρk,m∥2, the unit vector from element m to the UE,

$$\frac{\partial\delta_{k,m}}{\partial\mathbf{p}}
    =\mathbf{R}_k^{\mathsf T}\big[\mathbf{s}_{k,m}(\mathbf{p})-\mathbf{s}_{{\rm UR},k}(\mathbf{p})\big],
    \label{eq:apertjac}$$

the deviation of each element’s bearing from the panel bearing that [eq:delayjac] already carries. This tangent scales as $O(\|\boldsymbol{\rho}_{k,m}\|/d_{{\rm UR},k})$ and weakens toward the far-field limit. Whether it restores rank four is not settled by counting; it requires conditions on the RIS codes, wavefront curvature across the panel aperture and geometry. After complex-scale elimination the projected tangents Πhk⊥∂hk/∂x,∂hk/∂y,∂hk/∂z retain the phase slope and curvature of [eq:delta]. This rank test is local and conditional on the panel association. Pseudoinverse projectors cover the rank-deficient Jones blocks of [eq:g0rank]. The derivation of [eq:efim_short] is given in the supplementary material.

## C1: Maxwell–Jones Information Splitting

Both parts of C1 rest on one identity: because Ak is a Kronecker product, so is its projector, and the complement separates into a polarization-orthogonal component along the geometric factor and a geometry-orthogonal component acting identically on every polarization.

Lemma 1 (Kronecker projector splitting).  Let c≠0 and Θ≠0 be arbitrary, with PΘ=ΘΘ†. With A=c⊗Θ, ΠA⊥=Pc⊗ΠΘ⊥+Πc⊥⊗I,  Pc=ccH∥c∥2.

AHA=cHcΘHΘ and A†=c†⊗Θ†, so PA=Pc⊗PΘ. Expanding I=Pc+Πc⊥⊗PΘ+ΠΘ⊥ and subtracting PA gives [eq:kronsplit]. No rank assumption is used, so the identity also holds when a receiver mode makes the retained Jones block rank deficient.

Theorem 1 (C1: Maxwell–Jones information splitting).  (i) Information consequence. For one panel, write A=c⊗Θ, ζ=Θx, $\dot\mathbf{c}_i=\partial\mathbf{c}/\partial\chi_i$, and ζi=∂Θ/∂χix. Then $$_{ij}
 =\|\boldsymbol{\zeta}\|^2\dot\mathbf{c}_i^{\mathsf H}\boldsymbol{\Pi}_{\mathbf{c}}^\perp\dot\mathbf{c}_j
 +\|\mathbf{c}\|^2\dot{\boldsymbol{\zeta}}_i^{\mathsf H}
 \boldsymbol{\Pi}_{\boldsymbol{\Theta}}^\perp\dot{\boldsymbol{\zeta}}_j.
 \label{eq:split_short}$$ Since calibrated infrastructure gives ∂Θ/∂χ=0, the second term vanishes and $$\mathbf{J}_\chi^{\rm free}=\frac{2}{\sigma^2}\|\boldsymbol{\Theta}\mathbf{x}\|^2\,
    \operatorname{Re}\{\dot\mathbf{c}^{\mathsf H}\boldsymbol{\Pi}_{\mathbf{c}}^{\perp}\dot\mathbf{c}\},
    \label{eq:c1_scalarfac}$$ with $\dot\mathbf{c}=[\dot\mathbf{c}_1,\ldots,\dot\mathbf{c}_4]$: the polarimetric block enters only through the scalar ∥Θx∥2, and all χ-dependence is confined to the geometric factor. Throughout C1, Θ=Θk6 is the full block of [eq:Theta], and $\mathbf{S}_{\rm mode}$ is one of the selections in [eq:smode]. For a single panel with nonzero retained signal $\|\mathbf{S}_{\rm mode}\boldsymbol{\Theta}\mathbf{x}\|>0$, $$\begin{aligned}
    \mathbf{J}_\chi^{\mathbf{S}_{\rm mode}}&=\alpha_{\mathbf{S}_{\rm mode}}\mathbf{J}_\chi^{\rm full},
    \quad
    {\rm PEB}^{\mathbf{S}_{\rm mode}}
    =\alpha_{\mathbf{S}_{\rm mode}}^{-1/2}\,{\rm PEB}^{\rm full},\nonumber\\
    \alpha_{\mathbf{S}_{\rm mode}}&=\frac{\|\mathbf{S}_{\rm mode}\boldsymbol{\Theta}\mathbf{x}\|^2}
    {\|\boldsymbol{\Theta}\mathbf{x}\|^2}.
    \label{eq:c1_scaling}
\end{aligned}$$ The ${\rm PEB}$ identity additionally requires $\mathbf{J}_\chi^{\rm full}\succ\mathbf0$ so that both bounds are finite.

(ii) Discrimination consequence. Define $\mathbf{A}_k^{(\mathbf{S})}(\boldsymbol{\chi})=\mathbf{c}_k(\boldsymbol{\chi})\otimes
\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_k^{(6)}$. If a competitor block matches panel k’s space–frequency signature, i.e., ck′χ′∥ckχ, its free-Jones residual is $$\|\boldsymbol{\Pi}_{\mathbf{A}_{k'}^{(\mathbf{S})}(\boldsymbol{\chi}')}^\perp
 \mathbf{A}_k^{(\mathbf{S})}(\boldsymbol{\chi})\mathbf{x}_k\|
 =\|\mathbf{c}_k\|\,\|\boldsymbol{\Pi}_{\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_{k'}^{(6)}}^\perp
 \mathbf{S}_{\rm mode}\boldsymbol{\Theta}_k^{(6)}\mathbf{x}_k\|.
 \label{eq:panelres_short}$$ Writing Rkk′ for [eq:panelres_short], $$R_{kk'}\begin{cases}
    {}=0, & d_{\rm evs}\le2,\\[2pt]
    {}\ge\|\mathbf{c}_k\|\,\|\boldsymbol{\Theta}_k^{(6)}\mathbf{x}_k\|\sin\theta_{\min},
    & d_{\rm evs}=6
    \end{cases}.
    \label{eq:c1_dichotomy}$$ The first line assumes that the retained competitor block $\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_{k'}^{(6)}\in\mathbb{C}^{d_{\rm evs}\times2}$ has full row rank. The angle θmin is the minimum principal angle between the two panel Maxwell receive subspaces, and θmin>0 whenever they are distinct.

(i) A Jacobian column of Ax=c⊗ζ is $\mathbf{D}_i=\dot\mathbf{c}_i\otimes\boldsymbol{\zeta}
+\mathbf{c}\otimes\dot{\boldsymbol{\zeta}}_i$. Insert Lemma 1 into DiHΠA⊥Dj and apply the mixed-product rule: every cross term carries a factor Πc⊥c=0 or ΠΘ⊥ζ=0, leaving the two terms of [eq:split_short]. Applying a receiver mode replaces Θ by $\mathbf{S}_{\rm mode}\boldsymbol{\Theta}$ and leaves c untouched, so with ζi=0 the whole matrix scales by $\alpha_{\mathbf{S}_{\rm mode}}$.

(ii) Apply [eq:kronsplit] to the competitor Ak′Sχ′ and evaluate it at $\mathbf{c}_k\otimes\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_k^{(6)}\mathbf{x}_k$. Under ck′∥ck the term Πck′⊥⊗I annihilates its argument, leaving $\mathbf{P}_{\mathbf{c}_{k'}}\otimes\boldsymbol{\Pi}^{\perp}_{\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_{k'}^{(6)}}$ and hence [eq:panelres_short]. If $\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_{k'}^{(6)}\in\mathbb{C}^{d_{\rm evs}\times2}$ has full row rank with $d_{\rm evs}\le2$, its range is all of $\mathbb{C}^{d_{\rm evs}}$, and that projector is zero. For $d_{\rm evs}=6$, the two Maxwell receive subspaces are proper subspaces of C6 separated by a minimum principal angle θmin>0, so ∥ΠΘk′6⊥z∥≥sinθmin∥z∥ for every z∈colΘk6, which is the stated bound.

Remark 1 (Propagation angle and Maxwell separation).  For the ideal six-component block [eq:Theta], let $\cos\varphi_{kk'}=\mathbf{s}_{{\rm RB},k}^{\mathsf T}\mathbf{s}_{{\rm RB},k'}$. The minimum principal angle between the two Maxwell receive subspaces satisfies cosθmin=12∥Θk6HΘk′6∥2=1+cosφkk′2. To obtain this identity, choose transverse bases with a common second vector. Their cross Gram is 1+cosφkk′I2, while each self Gram is 2I2; changing either basis preserves the singular values. Equation [eq:maxwell_angle] relates the full-EVS discrimination term in [eq:panelres_short] to the RIS–BS angular separation.

The information scaling is per panel. If the panel blocks and their projected tangents are mutually orthogonal,

$$\mathbf{J}_\chi^{\mathbf{S}_{\rm mode}}=\sum_{k=1}^{K}\alpha_{\mathbf{S},k}\,
    \mathbf{J}_{\chi,k}^{\rm full},
    \qquad
    \alpha_{\mathbf{S},k}=\frac{\|\mathbf{S}_{\rm mode}\boldsymbol{\Theta}_k^{(6)}\mathbf{x}_k\|^2}
        {\|\boldsymbol{\Theta}_k^{(6)}\mathbf{x}_k\|^2}.
    \label{eq:c1_aggregate}$$

Outside this case, no single scalar generally rescales the aggregate.

Remark 2 (Scope of the free-Jones reference bound).  The free-Jones PEB is the bound of the unpenalized data model. Because the fixed λk=1 penalty does not vanish with σ, the penalized estimator’s departure from the free-Jones reference is measured rather than bounded: for the full-EVS mode, varying λk∈{0,0.1,1,10} changes the conditional position root-mean-square error (RMSE) by less than 2×10−5 in relative terms (supplementary).

## C2: Sufficient Maxwell–Kronecker Compression

Theorem 2 (C2: Sufficient Maxwell–Kronecker compression).  Let US=INT⊗QS be the semi-unitary lifting matrix in the vectorization of [eq:obs], and suppose colAχ⊆colUS for all admissible χ, which [eq:calibsub] establishes from calibration alone. Write yc=USHy and c⊥=∥I−USUSHy∥2. Under the matched white-Gaussian model: (i) for known σ2, yc is sufficient for χ,x; (ii) for unknown σ2, the pair yc,c⊥ is sufficient for χ,x,σ2, and yc alone still preserves the maximizer in χ,x of the σ2-profiled likelihood, because ∥y−Aχx∥2=∥yc−USHAχx∥2+c⊥, and c⊥ does not depend on χ,x; (iii) yc alone preserves the profiled optimizer, the derivatives of ∥y−Aχx∥2 in χ,x to every order, and the free-Jones EFIM [eq:efim_short] exactly; (iv) the noiseless delay poles are preserved provided the compressed receive–configuration snapshot matrix w1,…,wK, defined in [eq:freqsnap] by wk=ak⊗hk and ak=βkQSHBkek, has rank K.

Complete US to a unitary US,US⊥. The subspace inclusion makes US⊥Hy∼CN0,σ2I, independent of USHy and free of χ,x, which gives (i). For unknown σ2, its likelihood depends on the discarded block only through the scalar c⊥, so the factorization theorem gives (ii). Because A=USUSHA, the cross term in the orthogonal decomposition of y vanishes, which is [eq:isometry_short] with a data-only second term. Because US is parameter-independent, write A=USAc and Dχ=USDχ,c. Semi-unitarity then gives

$$\begin{aligned}
 \boldsymbol{\Pi}_{\mathbf{A}}^{\perp}\mathbf{D}_\chi
 &=\mathbf{U}_S\boldsymbol{\Pi}_{\mathbf{A}_c}^{\perp}\mathbf{D}_{\chi,c},\nonumber\\
 \mathbf{J}_\chi^{\rm free}
 &=\frac{2}{\sigma^2}\operatorname{Re}\{\mathbf{D}_{\chi,c}^{\mathsf H}
 \boldsymbol{\Pi}_{\mathbf{A}_c}^{\perp}\mathbf{D}_{\chi,c}\}
 =\mathbf{J}_{\chi,c}^{\rm free},
 \label{eq:efim_isometry}
\end{aligned}$$

proving (iii), including rank-deficient Jones blocks. For (iv), QSH acts on the EVS mode alone and commutes with both Vandermonde frequency factors, so the noiseless pole set is unchanged and the shift-invariance argument remains valid whenever the compressed snapshot matrix has column rank K.

Compression leaves the ML criterion unchanged but removes I−r noise-only receive directions from the ESPRIT front end. Its finite-sample acquisition benefit is assessed in Section 5.

## C3: Certified Common-Clock Profiling

Theorem 3 (C3: Certified common-clock profiling).  Let P and T be the compact search sets of Section 2.2 and assume $\mathbf{G}_\lambda(\mathbf{p})=\mathbf{G}_0(\mathbf{p})+\mathbf{R}_{\rm dir}(\mathbf{p})\succ\mathbf0$ on P. At each p, Qp,⋅ is the real trigonometric polynomial [eq:trig_short] of degree at most N−1, and the certificate below rests on its curvature bound [eq:clock_curvature].

(i) Exact nesting. For the penalized criterion, analytic Jones elimination followed by global clock maximization is exact: Fp,Δt=minxJp,Δt,x=∥y∥2−Qp,Δt, minp∈PFp=minp∈P, Δt∈T, x∈C2KJp,Δt,x, and the minimizer sets correspond: combining any outer minimizer p⋆ with a global inner clock maximizer Δt⋆ of Qp⋆,⋅ and x⋆=Gλ−1p⋆bp⋆,Δt⋆ recovers a joint minimizer.

(ii) Unit invariance. The outer profile is invariant under every invertible affine clock-unit reparameterization with T transformed accordingly.

(iii) Certified clock solution. At a visited position p, write qps=Qp,s, let Δt⋆p be a global clock maximizer and Δtp the incumbent. The interval bound [eq:bound_short] guarantees $$\begin{aligned}
 q_{\mathbf{p}}(\Delta t^\star(\mathbf{p}))-q_{\mathbf{p}}(\widehat{\Delta t}(\mathbf{p}))
 &\le g_{\rm cert},\nonumber\\
 g_{\rm cert}&=U-q_{\mathbf{p}}(\widehat{\Delta t}(\mathbf{p})),
 \label{eq:cert_short}
\end{aligned}$$ and $g_{\rm cert}\le\epsilon_{\rm clk}$ whenever termination occurs by the prescribed tolerance rather than the subdivision cap.

(i) Gλ≻0 makes [eq:jquad] a strictly convex quadratic in x, so the inner minimum ∥y∥2−Q of [eq:vp_short] is attained at every p,Δt; minimization may then be performed in any order, which is [eq:nestval_short], and the correspondence of minimizer sets follows from the same attainment. (ii) An invertible affine change of clock unit is a bijection of T onto its image, so the range of Qp,⋅, and hence F, is unchanged. (iii) The curvature bound L2 in [eq:clock_curvature] holds uniformly on T, so [eq:bound_short] is valid on every interval. The solver maintains a finite cover of T and records the upper bound for every interval it discards, so U dominates the bound for every interval in the cover and qpΔt⋆p≤U. This proves [eq:cert_short] under any stopping rule: the gap remains valid even when the subdivision cap is reached; only the conclusion $g_{\rm cert}\le\epsilon_{\rm clk}$ requires termination by tolerance.

Supplementary Lemma S1 gives a checkable anchor-column condition for Gλ≻0.

Theorem 3 solves the conditional clock problem globally and removes the clock from the outer coordinates; the remaining 3-D position search is local. Table 2 reports the clock-unit and certificate controls.

## Complexity

The implemented delay-subspace solver forms an N×N covariance and eigendecomposes it, at cost ON2rT+N3; a direct thin singular value decomposition (SVD) alternative costs ONrTmin{N,rT}. Let Fx,Fy be the zero-padded spatial FFT bin counts along the panel axes, $F_{\rm b}=F_xF_y$ their product, and Gr the number of beam-space trial ranges. Let $N_{\rm pk}$ and Nr denote the retained peaks and exact range samples. Generating candidates for all K2 panel–column pairs costs $O\!\left(K^2[TM_R+G_rF_{\rm b}\log F_{\rm b}
+N_{\rm pk}(N_rM_R+TM_R)]\right)$; the fixed two-permutation shortlist adds multi-start exact spherical refinements, while permutation arithmetic costs OKK!. The factorial term dominates asymptotically in K. For K=3 in the reference scene, beam-space processing and exact spherical-response evaluations have the leading operation count. A blockwise derivation and all fixed search budgets are supplementary. By Theorem 2, Phase II evaluates the exact forward model on the compressed observation, cutting residual and Jacobian rows from M=I N T to Mr=r N T without approximating the criterion. At Δt=0, the Kronecker structure gives

G0,kℓ=hkHhℓdkHdℓ BkHBℓ⏟cached,

so the Gram update costs Oq2N+T instead of Oq2Mr, with q=2K. For dense training matrices, generating hk=Ωkgk and its position derivatives costs OKTMR. Each outer position evaluation costs

$$\begin{aligned}
 C_{\rm outer}&=\underbrace{O(KTM_R+qM_r)}_{\text{responses and residual}}\nonumber\\
 &\quad+\underbrace{O\big(q^2(N+T)\big)}_{\text{Gram}}
 +\underbrace{O(q^3+q^2N^2)}_{\text{factorization and coefficients}}\nonumber\\
 &\quad+\underbrace{O(qM_c\log M_c)}_{\text{clock FFT}}+C_{\rm cert},
    \label{eq:cost}
\end{aligned}$$

with Mc the zero-padded clock-FFT length and $C_{\rm cert}$ the branch-and-bound cost. Tightening $\epsilon_{\rm clk}$ increases interval work inside $C_{\rm cert}$, with ON polynomial arithmetic per visited interval and no further Gram factorization.

# Numerical Results

## Simulation Setup

Reference-scene parameters. The panel frame sets its local z-axis along nk, its local x-axis along the normalized vector uz×nk and its local y-axis by right-handed completion; these axes determine Rk. Gains and Jones states are redrawn independently per trial, except for the analytically fixed Jones states in the controlled separability sweep of Fig. [fig:c1_separability].

| Parameter | Value |
| --- | --- |
| Carrier | fc=60 GHz, λc=5.00 mm |
| RIS panels | K=3, MR=64×64, λc/2 spacing |
| BS array | MA=16 full-EVS ULA, λc/2 spacing |
| OFDM | N=63, Δf=5 MHz, balanced Hankel lift with P=L=32 |
| Training | T=256 configurations; block-static UE, clock, gains and Jones states |
| UE / box [m] | 1.25,0.55,0.75; 0.30,2.70​×​−1.40,1.50​×​0.35,1.45 |
| BS [m] | −22.50,−12.50,2.00 |
| RIS centers rk [m] | 4.20,−2.20,1.05; 5.10,2.10,1.15; 4.80,0,1.25 |
| RIS normals nk | −0.9159,0.4005,−0.0264; −0.8523,−0.5205,−0.0528; −0.9940,0.0158,−0.1084 |
| Clock | $\Delta t_{\rm true}=5$ ns; T=0,10 ns |
| Gains | $\beta_k\stackrel{\rm iid}{\sim}\mathcal{CN}(0,1/K)$ |
| Jones states | ek=sinγkejψk,cosγkT, γk​∼​U0.35,1.10, ψk​∼​U(−π,π] |
| Trials | 480 per accuracy cell; 960 per benchmark cell |

In the reference scene of Table 1, $\max_k d_{{\rm UR},k}=4.2~{\rm m}<R_{\rm FF}=19.8~{\rm m}
<\min_k d_{{\rm RB},k}=28.6$ m, satisfying (A3) with $D_{\rm ap}=0.223$ m. Writing μ=Aχx for the noiseless observation and $\boldsymbol{\mu}_{\rm act}$ for its restriction to the $M_{\rm act}$ entries carried by the receive channels that $\mathbf{S}_{\rm mode}$ retains, the operating point is set by

$${\rm SNR}=10\log_{10}\frac{\|\boldsymbol{\mu}_{\rm act}\|_2^2}
    {M_{\rm act}\sigma^2},
    \label{eq:snrdef}$$

which defines the average per-sample signal-to-noise ratio (SNR), with absolute path loss absorbed into the normalization.

For R trials, let $\mathbf{p}_{\rm true}$ and $\Delta t_{\rm true}$ denote the generating position and clock, set $e_{p,r}=\|\hat\mathbf{p}_r-\mathbf{p}_{\rm true}\|_2$ and $e_{t,r}=|\widehat{\Delta t}_r-\Delta t_{\rm true}|$, and let ${\rm fail}_r$ denote the event that a route terminates without returning an estimate. The position and clock catastrophic rates are

$$\begin{aligned}
    P_{\rm cat}^{(p)}&=\frac1R\sum_{r=1}^{R}
    \mathbf 1\{{\rm fail}_r\ \lor\ e_{p,r}>0.1~{\rm m}\},\nonumber\\
    P_{\rm cat}^{(t)}&=\frac1R\sum_{r=1}^{R}
        \mathbf 1\{{\rm fail}_r\ \lor\ e_{t,r}>1~{\rm ns}\}.
    \label{eq:pcat}
\end{aligned}$$

Position and clock conditional RMSEs are each taken over the complement of their own event in [eq:pcat]; the Phase-II control of Table 2 instead reports both on a single per-route non-catastrophic set, as its note states. A Phase-I basin-acquisition event occurs on trial r when $\hat\mathbf{p}_{{\rm I},r}$ is finite and $\|\hat\mathbf{p}_{{\rm I},r}-\mathbf{p}_{\rm true}\|_2\le0.1$ m. The declared operating region is ${\rm SNR}\ge-10$ dB; results at −15 or −20 dB are labeled as below it. Every cell generates observations and performs estimation on the same center-frequency spatial manifold, so $\epsilon_{\rm sq}$ of Section 2.2 contributes no residual. In contrast, a nine-sample per-subcarrier diagnostic produces a deterministic offset of at most 0.080 mm in position, 0.171 of that sample’s −10-dB PEB, but up to 5.4 ps in clock, exceeding the sample-specific −10-dB CEB in four of the nine samples. Within each cell, all methods share the scene, RIS codes, Jones states, gains and noise.

## End-to-End Benchmark

image

Fig. [fig:benchmark_combined] compares five end-to-end routes using shared realizations. The closest in-model control, baseline-initialized 4D-JVP, pairs the same exact forward model with a frozen baseline Phase-I initializer and an explicit range-scaled 4-D Jones variable projection. The proposed route pairs MKSC-GI acquisition with CCOP-JVP, so this end-to-end comparison does not isolate C3. Their −10 dB median position errors differ by only 0.006 mm, but the proposed route lowers the position and clock catastrophic rates by 6.0× and 6.7× (annotated in Fig. [fig:benchmark_combined](c) and (d)).

For algebraic canonical polyadic decomposition (CPD) , the median error decreases as SNR increases, while the catastrophic rate remains near 30%. Variational Bayesian inference with sparse Bayesian learning (VBI/SBL)  lowers its catastrophic rate with SNR yet retains a 26.1-mm median throughout. The route combining near-field orthogonal matching pursuit, space-alternating generalized expectation-maximization and weighted least squares (OMP-SAGE-WLS)  is flat in both metrics when adapted to the shared random training. At −10 dB the proposed route lowers both position and clock catastrophic rates by more than twentyfold relative to the best external adaptation. For the paired catastrophic indicators, McNemar’s exact test rejects equality with each of the four alternatives (p≤1.91×10−6). These comparisons use each adaptation’s native readout and characterize the adaptations, not the cited methods. In particular, the OMP-SAGE-WLS comparison uses the cited method’s regular-RIS route; its sub-RIS variant requires orthogonal phase profiles  that the shared random training cannot supply. Baseline specifications and adaptations are supplementary.

## C2: Sufficient Compression and Acquisition

The single-factor test isolates projection before delay-subspace estimation. σj denotes the jth singular value of the forward–backward augmented tone–snapshot matrix used by that estimator. At −10 dB, delay-subspace estimation from the raw 96-dimensional observation gives leakage ratio σK+1/σK=0.824 and a 2.50% position catastrophic rate. After retaining six receive dimensions, the ratio is 0.357 and no catastrophic events are observed (0/480). At 0 dB and above, the compressed and uncompressed routes coincide.

The independently seeded ladder in Table [tab:c2_ladder] separates the remaining Phase-I changes at the −10-dB operating boundary and at −20 dB, below it. At −10 dB, the raw-EVS row has $P_{\rm cat}=2.92\%$; compression leaves no observed events (0/480), and common-geometry fusion brings basin acquisition from 99.17% to 100%. The −20-dB rows show 20.42% catastrophic events with raw EVS and 6.04% after compression. Common-geometry fusion changes basin acquisition from 76.88% to 93.96%; four deterministic starts yield $P_{\rm cat}=4.17\%$ and 95.21% basin acquisition. At this SNR, anchor refresh does not change either event count but reduces the 95th percentile (p95) of reconstruction normalized mean-square error (NMSE), computed against the noiseless I×N×T receive–tone–training tensor, from 0.0953 to 0.00112. The oracle control in Table [tab:c2_ladder] uses the true UE position for anchor refresh and fixes only the CCOP-JVP position initializer at that value; the true clock is not supplied.

In the panel-size sweep, the proposed route’s conditional RMSE is 2.02 mm at 162 elements per panel and 0.43 mm at 642; its position catastrophic rate stays at or below 1.04%. Across this sixteenfold change in element count, proposed Phase I basin acquisition remains above 98.9%, while baseline Phase I rises from 14.79% to 95.83%.

lcccc & &
(lr)2-3(lr)4-5 Phase-I route & $P_{\rm cat}\!\downarrow$ & Basin acq.↑ & $P_{\rm cat}\!\downarrow$ & Basin acq.↑
Baseline (raw EVS) & 2.92 & 96.04 & 20.42 & 63.12
+ MKSC compression & 0.00 & 99.17 & 6.04 & 76.88
& 0.00 & 100.00 & 5.42 & 93.96
+ 4 det. starts & 0.00 & 100.00 & 4.17 & 95.21
+ anchor refresh & 0.00 & 100.00 & 4.17 & 95.21
Oracle control & 0.00 & 100.00 & 0.00 & 100.00

## C3: Certified Clock Profiling

Table 2 holds 100 MKSC-GI initializers fixed across Phase-II routes. CCOP-JVP and the range-scaled c0Δt 4D-JVP control have the same conditional position and clock RMSE to the reported precision. Profiling leaves three outer coordinates instead of four, with median evaluations of 9 versus 13 for the c0Δt control. CCOP-JVP also returns bit-identical outputs under all three unit-scaled parameterizations, as Theorem 3(ii) predicts. The explicit-clock control is unit sensitive: expressing its clock coordinate in seconds increases conditional position RMSE by 32% relative to c0Δt. All 950 conditional clock profiles meet the prescribed objective-gap tolerance with no subdivision-cap hit.

Phase-II refinement control at −10 dB over 100 shared MKSC-GI initializers; the three MKSC-GI-initialized 4D-JVP columns differ only in clock parameterization.

|  | 4D-JVP clock coordinate |  |  | CCOP-JVP |
| --- | --- | --- | --- | --- |
| 2-4 Metric | Seconds | ns | c0Δt | Profiled |
| Position RMSE [mm] | 0.6435 | 0.4845 | 0.4869 | 0.4869 |
| Clock RMSE [ps] | 3.9926 | 3.5970 | 3.5855 | 3.5855 |
| $P_{\rm cat}$ [%] | 1 | 0 | 1 | 1 |
| Outer dimension | 4 | 4 | 4 | 3 |
| Median evaluations | 13 | 18 | 13 | 9 |
| Unit-invariant | no | no | no | yes |
| Inner clock certificate | no | no | no | yes |
| Both RMSEs of a column are taken over that route’s non-catastrophic set, the trials triggering neither event in [eq:pcat], rather than over the per-metric complements of Section 5.1. The certificate covers 950/950 inner clock profiles, not the outer search. |  |  |  |  |

Conditional position RMSE against the free-Jones PEB, 480 paired trials per cell at a common absolute noise level.

## C1: Receiver Information and Panel Separability

The three receiver modes are compared at a common absolute noise level, $\sigma^2_{\rm scalar}=\sigma^2_{\rm dual}=\sigma^2_{\rm full}$ set from the full-EVS observation, rather than at a common post-selection SNR. Fig. 1 compares conditional accuracy with the matched multi-panel PEB: at 0 dB the scalar, dual-polarized and full-EVS modes reach 0.751, 0.296 and 0.131 mm, ordering exactly as the matched PEB predicts. Theorem 1(i) separately identifies the scalar information weight $\alpha_{\mathbf{S}_{\rm mode}}$ for a single panel.

Fig. [fig:c1_separability] sweeps the Jones overlap ekHek′ of the closest-delay panel pair against cascaded-delay separation and scores resolution probability—the fraction of trials returning K correctly associated components with Phase-I delay error at most 0.5 ns and no pole collapse—rather than position accuracy. For this sweep, the panel-scaling sweep and Fig. [fig:boundaries](c), the candidate set is the union of the beam-space proposals and the fixed-count dictionary of Section 3.2, whose size is specified in the supplementary material; every other cell uses the proposals alone. Full-EVS resolution is flat across the four overlaps, varying by less than the Clopper–Pearson half-width of a single cell. The dual-polarized and scalar modes collapse toward zero as the overlap shrinks, with maximum spreads across overlaps of 82.9 and 63.5 percentage points. As the competing panel’s Jones state rotates away from that of the reference panel, it projects increasingly onto a weakly retained Maxwell direction, so the scalar weight of Theorem 1(i) becomes panel-dependent: under the dual-polarized mode $\alpha_{\mathbf{S}_{\rm mode}}$ remains at 0.50 for the reference panel while falling to 0.005 for the competing one at the smallest overlap; under the full-EVS mode, it equals one for both panels at every overlap. With $d_{\rm evs}\le2$ the competing Jones block spans $\mathbb{C}^{d_{\rm evs}}$, so by [eq:c1_dichotomy] the residual separating two panels can vanish outright; with six components it is bounded below by sinθmin. Of the 8 429 association failures in the 34 560 sweep trials, 8 351 coincide with a Phase-I delay failure. The resolution probability therefore reflects both delay acquisition and panel association. At −10 dB, the three receiver modes have position catastrophic rates of 13.33%, 1.88% and 0.83%.

image

image

## Robustness and Deployment Limits

Among the tested ranges, RIS–BS calibration is the most restrictive (Fig. [fig:boundaries](a)–(b)): angular error degrades precision before it degrades acquisition reliability, raising conditional RMSE from 0.416 to 31.1 mm at 0.25∘ even though the basin-acquisition rate remains 99.58%; at 1∘, the position catastrophic rate reaches 58.54%.

In the dedicated 0-dB model-order sweep, underestimating K with K=2 gives a 0.705-m median and $P_{\rm cat}=65.6\%$; overestimating it with K=4 or 5 preserves submillimeter medians but yields position catastrophic rates above one third.

Delay coincidence limits acquisition even where the local EFIM remains nonsingular (Fig. [fig:boundaries](c)). Across a 5×5×2 position grid, the proposed median per-position catastrophic rate is 0.62% against 2.71% for baseline-initialized 4D-JVP. The two worst grid corners lie near a cascaded-delay coincidence surface; the worst reaches 29.58% at a minimum separation of $0.062/B_{\rm occ}$ (0.197 ns), while the free-Jones EFIM remains full rank at every tested position, including those corners. Deployment screening should include a delay-coincidence check and a coarse global fallback in addition to a PEB map.

A 25% cyclic prefix at Δf=5 MHz lasts 50 ns and covers the 19.1-ns delay span over the clock-search interval. With one configuration per 250-ns OFDM symbol, T=256 configurations span about 64 s, the block-static interval required by (A6).

# Conclusion

Receiver-mode selection and cascaded-delay coincidence expose acquisition risks that the free-Jones EFIM does not exclude. A reduced receiver can make a panel pair difficult to resolve even when a single-panel free-Jones EFIM remains finite. At the worst tested position-grid point, the multi-panel EFIM is full rank but the position catastrophic rate is 29.58%. Deployment checks need receiver-separability and delay-coincidence assessments alongside a PEB map.

MKSC-GI projects out noise-only receive directions before delay acquisition and fits a common panel geometry. CCOP-JVP profiles the clock at fixed position and uses a deterministic objective-gap certificate for its inner solution. In the controlled Phase-II comparison, it matches the explicit-clock route’s RMSE while reducing the outer dimension from four to three. For the calibrated reference scene with known K and the matched center-frequency spatial model, the −10-dB end-to-end median position error is 0.32 mm; position and clock catastrophic rates are 0.42% and 0.31%. Scaling to larger panel counts calls for association without factorial permutation search.